Code Example 1: Linear MPC for building thermal dynamics

Code: week_9_example_1.py

Problem description:

Reference tracking formulation 
dynamics constraints given in a dense form (single shooting)

The code defines a linear discrete-time state-space model of a building with 4 thermal states (e.g., floor, walls, facade, and internal air temperature):

- A: system dynamics matrix

- B: control input matrix (for HVAC power)

- E: disturbance matrix (external temp, occupancy, solar)

- C: output matrix selecting the internal temperature as the output to control

The model evolves as:

xk+1=Axk + Buk + Edk

yk​=Cxk​

At every time step, the code solves an MPC optimization problem to determine the HVAC power input u_k over a prediction horizon N (e.g., 10 steps). The goal is:

- Track a temperature setpoint (e.g., 21°C)

- Minimize:

  - Output tracking error (yk−yref)2

  - Control effort (uk)2

  - Slack variable penalties (if output exceeds comfort range)

Constraints include:

- HVAC input limits (e.g., 0–6000 W)

- Output (temperature) comfort bounds: [20, 22] °C (soft constraints via slacks)

Code Example 2: Linear MPC for building thermal dynamics

Code: week_9_example_2.py

Problem description:

Modification of example 1

Zone tracking formulation - economic MPC objective
dynamics constraints given in a dense form (single shooting)

Code Example 3: Linear MPC for building thermal dynamics

Code: week_9_example_3.py

Problem description:

Modification of example 1

Reference tracking formulation 
dynamics constraints given in a dense form (single shooting)
dynamics with disturbances (external temperature, occupancy, solar gain)

Code Example 4: Linear MPC for building thermal dynamics

Code: week_9_example_4.py

Problem description:

Modification of example 2

Zone tracking formulation - economic MPC objective
dynamics constraints given in a dense form (single shooting)
dynamics with disturbances (external temperature, occupancy, solar gain)

Code Example 5: MPPI Control for the building thermal model

Code: week_9_example_5.py

Problem description: modification of example 3

Optimization-free Sampling-based MPPI solution